| Evaluation Sheet: 5-284 | Horizontal Ventilation |
| --- | --- |

| Objective: You are the #4 FF on a Ladder Company that has arrived as the first due truck at a fire in a 2-story house. Your officer tells you that the company is going to search and vent. You are tasked with performing ventilation and providing egress. Complete in SFD time goal of 9:40. Do you have any questions? |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Standard: NFPA 1001, 2019 Edition 4.2.3, 4.3.4, 4.3.6, 4.3.11, 4.3.18 |  | Reference: BSM CH.5, Skills Sheets: 2-1, 5-33, 5-44, 5-45, 5-80. |  |  |  |  |
| Task: Communicate the fires location, check the basement, provide horizontal ventilation, and ensure egress. |  |  |  |  |  |  |
| Performance Outcome: The candidate shall be able to provide coordinated ventilation, force entry, remove window bars, and shut off gas to building. |  |  |  |  |  |  |
| Conditions: Given full PPE (radio, turnouts, SCBA), 28’ ladder, halligan, NY hook, forcible entry saw, PPV fan, and 24’ ladder, the candidate will: |  |  |  |  |  |  |
| No. | Task Steps |  | First Test |  | Retest |  |
|  |  |  | Pass | Fail | Pass | Fail |
|  | Access Side Charlie. TIME STARTS: (when ladder is touched) - Carry the ladder and tools to side Charlie and identify the fire’s location. - Place the ladder to floor 2 for egress from the most threatened bedroom. - Radio Command: “Command from L90 Team Bravo, there is an unvented fire, floor 1 bedroom on the Charlie/Delta corner.” - Cover (no skin visible, flaps down). |  |  |  |  |  |
|  | Check the basement. - Use the baseball swing method to force an inward swinging back door. - Sweep for victims inside doorway and close the door. - Radio Command: “Command from L90 Team Bravo, there is no fire in the basement.” |  |  |  |  |  |
|  | Ventilate for fire (AFTER E90 has called “water on the fire”). - FMR must be connected prior to performing the VENT. - Clear the glass and then the mullion from the fire room window (without standing directly in front of the vent point). - Radio Command: “Command from L90 Team Bravo, ventilation point established.” |  |  |  |  |  |
|  | Positive Pressure Ventilation (establish AFTER “water on the fire” or steam conversion visible and no contraindications of PPV). - Size-up for PPV. “There is no fire in the void spaces”. - Start PPV fan facing away from entry path. Check for and state “good steam conversion” coming from vent point. - Direct the fan into the door the Engine entered at idle, slowly increase the fan to full throttle, and check seal on the door with exposed skin. - Evaluate the effectiveness of PPV at the vent point and state “good smoke removal” and check the eaves for any indications of hidden fire. - Radio Command: “Command from L90 Team Bravo, PPV is established.” |  |  |  |  |  |

|  | “Walk the path” with partner to ensure effectiveness, expand vent points as needed. - Perform “buddy check” with partner. - Ensure open pathway between entry door and vent point. - Compartmentalize building. - Expand ventilation exit points as needed. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Shut off the natural gas. |  |  |  |  |
|  | Ensure Egress. - Place a ladder to floor 2-bedroom window on side Alpha for egress. - Use the FE saw to cut window bars. - Use the gap-set-force method to force an inward swinging door (must use all techniques in order). |  |  |  |  |
|  | Close door on a tool. TIME STOPS: |  |  |  |  |
|  | Conduct all procedures in a safe and controlled manner. |  |  |  |  |
| SFD Time Goal: N/A |  |  |  |  |  |

| Instructor/Candidate Comments: |  |
| --- | --- |

| Instructor (Print & Sign) |  | Date |  | Candidate |  | Date |
| --- | --- | --- | --- | --- | --- | --- |
| Retest: |  |  |  |  |  |  |
| Instructor (Print & Sign) |  | Date |  | Candidate |  | Date |