| Evaluation Sheet: 2-201 | Sling SCOTT AIR-PAK X3 |
| --- | --- |

| Objective: Demonstrate proficiency donning an SCBA prior to entering an IDLH fire environment. The time standard is 1:00. Do you have any questions? |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Standards: WAC 296-305-04001 (15) NFPA 1001 2019 Edition 4.1.2, 4.3.1 |  |  | Reference: Scott Safety Operating and Maintenance Instructions |  |  |  |  |
| Task: Sling SCBA and go on air. |  |  |  |  |  |  |  |
| Performance Outcome: The candidate will successfully demonstrate the ability to properly don an SCBA. |  |  |  |  |  |  |  |
| Conditions: Given a SCOTT AIR-PAK X3 SCBA, and full PPE (radio, turnouts, hood, helmet, gloves) the candidate will: |  |  |  |  |  |  |  |
| No. | Task Steps |  |  | First Test |  | Retest |  |
|  |  |  |  | Pass | Fail | Pass | Fail |
|  | Pressurize system TIME STARTS: Time starts when instructor states “Go”. - Turn cylinder completely on. - Check and state “amount of air” in cylinder. - Confirm VIBRALERT activation and PAK-ALERT startup, state “PASS”. |  |  |  |  |  |  |
|  | Sling the AIR-PAK X3. - Use coat method over left shoulder. - Do not wildly swing in a manner that will damage any equipment. |  |  |  |  |  |  |
|  | Secure AIR-PAK X3. - Tension shoulder straps. - Connect and tension waist straps. |  |  |  |  |  |  |
|  | Cover. - Remove helmet (helmet may be placed behind the head, over the arm or placed on the ground in a controlled manner). - Secure facepiece against face. - Pull head net on and tighten all straps, starting from the bottom. - Check seal, use exposed hand and state “SEAL”. |  |  |  |  |  |  |
|  | Hood and helmet. - Pull hood into place with no skin or straps visible. - Replace helmet and snug chin strap. - Pull down ear flaps. - Connect FMR to facepiece and breathe air. - TIME STOPS: (after gloves are removed from right pocket and held in the air). |  |  |  |  |  |  |
|  | Gloves. - Don both firefighting gloves. |  |  |  |  |  |  |
| Retest Approved By: |  | SFD TIME GOAL: 1:00 |  |  |  |  |  |

| Instructor/Candidate Comments: |  |
| --- | --- |

| Instructor (Print & Sign) |  | Date |  | Candidate |  | Date |
| --- | --- | --- | --- | --- | --- | --- |
| Retest: |  |  |  |  |  |  |
| Instructor (Print & Sign) |  | Date |  | Candidate |  | Date |