| Evaluation Sheet: 3-203 | Pre-connect/V-Split #4 (Heel) |
| --- | --- |

| Objective: You are the #4 (Heel) on an Engine Company that has arrived as the first due Engine at a vented, Fully Developed fire on floor 2 of a house. Your officer tells you “Pre-Connect to the Alpha side, front door.” You must control the door, advance to the fire room with the #3, establish a vent point, complete a primary search, and communicate benchmarks to command. Complete in SFD time goal of 6:00. Do you have any questions? |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Standard: NFPA 1001, 2019 Edition 4.3.1, 4.3.2, 4.3.9, 4.3.10, 4.3.11, 4.3.13 |  | Reference: Hose Handling And Management |  |  |  |  |
| Task: Control the door, advance to the fire room, vent, search, and communicate the benchmarks. |  |  |  |  |  |  |
| Performance Outcome: The candidate shall be able to control the door, advance to the fire room, vent, search, and communicate benchmarks using proper radio procedures. |  |  |  |  |  |  |
| Conditions: Given full PPE and appropriate equipment, seat belted in the #4 seat, door closed, the candidate will: |  |  |  |  |  |  |
| No. | Task Steps |  | First Test |  | Retest |  |
|  |  |  | Pass | Fail | Pass | Fail |
|  | Don all appropriate PPE and equipment. |  |  |  |  |  |
|  | Safely exit the apparatus: Check and state: “Traffic.” |  |  |  |  |  |
|  | Take irons and perform a size up of building, (see 3 sides of bldg.) Advance to the front of the building. |  |  |  |  |  |
|  | Verbalize, “Delta side, Floor 2, vented fire smoke and flame showing” |  |  |  |  |  |
|  | Cover (no skin, flaps down, FMR attached, chin strap tight, gloves on.) - Assist with removing significant kinks from attack line as needed (less than 90°). |  |  |  |  |  |
|  | Buddy Check: Ensure partner is fully covered when asked for a “Buddy Check” |  |  |  |  |  |
|  | Door Ops - Ensure access, Force entry if required. - Open the door when both #3 (Nozzle) and #4 (Heel) are ready for entry. - Open and control the door with a tool, look for 4L’s: Life, Location, Layout, and Lift - Communicate with #3 on the conditions found. - Chock the door - Ensure door remains chocked open after the #3 passes the entry. |  |  |  |  |  |
|  | Advance to the fire room behind #3 (Nozzle) - Bring a bight of hose to base of stairs to support #3 hose line advancement. - Feed hose up the stairs, keeping hose low and straight behind #3. - Radio using Convey method: “Command from Engine ___, Team Bravo, water on the fire” When in position to contain the fire and #3 states at the fire room. - Be ready to use NFA handshake if not answered. |  |  |  |  |  |
|  | Bump-up, Peel off, Search - Peel off on command from the #3 (Nozzle) to search the fire room. - Complete a primary search. - Rescue any found victims. - Communicate fire behind to #3 (Nozzle) - Radio using Convey method: “Command from Engine ___, Team Bravo, Fire Under Control, primary search of the fire room complete, vent point established” |  |  |  |  |  |
|  | Complete all drill objectives in a safe manner. |  |  |  |  |  |
| SFD time goal: 6:00 |  |  |  |  |  |  |

| Instructor/Candidate Comments: |  |
| --- | --- |

| Instructor (Print & Sign) |  | Date |  | Candidate |  | Date |
| --- | --- | --- | --- | --- | --- | --- |
| Retest: |  |  |  |  |  |  |
| Instructor (Print & Sign) |  | Date |  | Candidate |  | Date |